Serial Command Manual
Phased Array Antenna Control Interface
(Customer Version)

General Information

| Item | Value |
| --- | --- |
| Baud Rate | 460800 |
| Data Bits | 8 |
| Parity | None |
| Stop Bits | 1 |
| Frame Format | A <CMD> <PARAM> Z |
| Response Format | ASCII text via printf() |

All commands start with 'A' and end with 'Z'. Each command execution returns an ASCII response.

Command : AIZ

Class: System Control

Function: Initialize System

Usage: Initialize all antenna parameters to default calibrated state.

Parameters

| Parameter | Description | Data Type |
| --- | --- | --- |
| I | Initialize system | String |

Example

Command:

AIZ

Response:

OK

Command : AT<m>Z

Class: System Control

Function: Select Operation Mode

Usage: Select RF operation mode.

Parameters

| Parameter | Description | Data Type |
| --- | --- | --- |
| m | Operation mode index | Integer (1 digit) |

Valid Values

| Value | Meaning |
| --- | --- |
| 0 | Standby |
| 1 | TX-V |
| 2 | TX-H |
| 3 | RX-V |
| 4 | RX-H |
| 5 | Sleep |

Example

Command:

AT1Z

Response:

OK,MODE=1

Command : AG<dd>Z

Class: RF Control

Function: Set Attenuation Step

Usage: Set overall RF attenuation step.

Parameters

| Parameter | Description | Data Type |
| --- | --- | --- |
| dd | Attenuation(dB) | Integer (2 digits) |

Valid Values

| Value | Meaning |
| --- | --- |
| dd = 00–15 | 00–15 dB attenuation |

Example

Command:

AG05Z

Response:

OK,ATT=5

Command : AB<y><x><sign><tt><ppp>Z

Class: Beamforming Control

Function: Update Beam Scan Angle

Usage: Update antenna beam direction using theta and phi.

Parameters

| Parameter | Description | Data Type |
| --- | --- | --- |
| y | Pre Y-index | Integer (1 digit) |
| x | Pre X-index | Integer (1 digit) |
| sign | Theta sign (+/-) | Char |
| tt | Theta angle | Integer (2 digits) |
| ppp | Phi angle | Integer (3 digits) |

Valid Values

| Value | Meaning |
| --- | --- |
| tt = (-90) – 90 | -90 to +90 deg |
| ppp = 0 – 359 | 000 to 359 deg |

Example

Command:

AB01+20300Z

Response:

OK,THETA=20,PHI=300

Command : AS<rr><data4:1>Z

Class: Low-Level Control

Function: Sub-chain Data Transfer

Usage: Transfer raw bitstream to selected sub-chain row.

Parameters

| Parameter | Description | Data Type |
| --- | --- | --- |
| rr | Sub-chain row index | Integer (2 digits) |
| datan | 60 bits SPI bitstream (0/1) | ASCII string |

Valid Values

| Value | Meaning |
| --- | --- |
| rr = 00 – 04 | 01 – 04 row |

Example

Command:

AS01<data of IC4><data of IC3><data of IC2><data of IC1>Z

Response:

OK,ROW=1

Command : AMZ

Class: Monitoring

Function: Read Board Temperature

Usage: Read on-board temperature sensor.

Parameters

| Parameter | Description | Data Type |
| --- | --- | --- |
| M | Read temperature | String |

Example

Command:

AMZ

Response:

TEMP=36.5C

Command : AQZ

Class: Beam Tracking

Function: Run Beam Tracking

Usage: Execute beam tracking algorithm.

Parameters

| Parameter | Description | Data Type |
| --- | --- | --- |
| Q | Start beam tracking | String |

Example

Command:

AQZ

Response:

OK,BEAM_TRACKING

Command : AC<sss>Z

Class: Calibration Control

Function: Apply Calibration State

Usage: Load and apply predefined calibration or beam state.

Parameters

| Parameter | Description | Data Type |
| --- | --- | --- |
| sss | State index | Integer (3 digits) |

Valid Values

| Value | Meaning |
| --- | --- |
| 000 | Initialize system |
| 001–999 | Apply calibration state |

Example

Command:

AC005Z

Response:

OK,STATE=5